# Physical Interaction Learning Journal – Matthew Seymour

Week 1 - 14/09/2026

Intro slides to physical interaction. Learnt basic stuff about breadboards,  resistors and the Arduino Uno. Made a light blink on and off on the Arduino to spell out our names in morse code.

Week 2 - 21/09/2026

Made the spaceship exercise with the three lights, which change which lights blink while holding down a button. Also learnt about potentiometers and the analog inputs. Did an exercise that used a potentiometer to control the rate at which a LED would blink on and off.

Week 3 - 28/09/2026

Learnt about the piranha RBB LED, which allows you to get any colour by using different amounts/combinations of red, green, and blue. Also used a potentiometer to change the colour. Learnt about light dependant resistors (LDR’s) which change their output value depending on how much light it detects. Also learnt about debouncers which are used to filter out buggy/irregular readings.

Week 4 - 05/10/2026

Learnt about Ultra Sonic Sensors which measure the distance between it and the thing it’s pointing at. It works by sending/transmitting a high frequency sound wave out, and measuring the time it takes for the soundwave to come back and hit the receiver. This amount of time is the amount of time it takes for the soundwave to go to the object and back again so you halve that value to get the amount of time it takes to just hit the object. Then you use the (distance / speed = time) equation to work out the distance.